ROADWAY TYPICAL

Scales Creek Rd

(BEG @ SR 51 TO +/- 500 FT past TOWN OF HOMER CL)

(ADD 1FT. WIDENING RIGHT SIDE)

ROADWAY WIDTH VARIES

MAINTAIN CROSS- SLOPE ¼” cross-slope per ft

ADJUST CL 6” RIGHT TO PROVIDE +/-11‘ LN

+/- 2’ SHOULDER FILLING							+/-2’ SHOULDER FILLING

SHOULDER VARIES					SHOULDER VARIES

NOT TO SCALE

RESURFACED AREA

6-7’ WIDE MILLED AREA, EXTEND 1 FT  OUTSIDE EXISTING PAVEMENT EDGE

19 MM-220LBS/SY

25 MM-440LBS/SY

EXISTING PAVEMENT

PAVEMENT

RETAIN EXISTING PAVEMENT AND MAINTAIN PROPER CROSS SLOPE. MILL 6’-7’ WIDE, EXTEND 1’ WIDENING OUTSIDE EXISTING PAVEMENT EDGE RIGHT SIDE WITH 25MM SUPERPAVE (440 LBS PER SQUARE YARD) & 19MM SUPERPAVE (220 LBS PER SQUARE YARD) .  PATCHING AND LEVELING 110 LBS PER SQUARE YARD OF OGI (FULL WIDTH) OR AS DIRECTED BY ENGINEER. RESURFACE FULL WIDTH WITH 135 LBS PER SQUARE YARD RECYCLED ASPHALTIC CONCRETE 9.5mm SUPERPAVE, GP 2 ONLY, INCL BITUM MATL & H LIME.

****ALL PATCHING WILL BE 4” IN DEPTH OR AS DIRECTED BY ENGINEER.

****SHOULDER FILLING AND GRASSING WILL BE PERFORMED BY CONTRACTOR.

****WIDENING DETAIL BELOW****

= 440 LBS PER SQUARE YARD RECYCLED ASPHALTIC CONCRETE 25 MM SUPERPAVE, GP 1 OR 2, INCL BITUM MATK & H LIME & 220 LBS PER SQUARE YARD RECYCLED ASPHALTIC CONCRETE 19 MM SUPERPAVE, GP 1 OR 2, INCL BITUM MATK & H LIME

1’- 0’

LT. OR RT.

110 LBS PER SQUARE YARD, OGI LEVELING

Scales Creek Rd