CONTRACT SPECIFIC SPECIAL PROVISIONS

- CONTRACTOR SHALL BE RESPONSIBLE FOR ALL MATERIAL TESTING AND MIX DESIGNS AND PROVIDING APPROVED JOB MIX FORMULAS FOR ALL ASPHALT. ALL TESTING FREQUENCIES AND METHODS MUST MEET THE MINIMUM REQUIREMENTS SET FORTH IN THE GEORGIA DEPARTMENT OF TRANSPORTATION STANDARD SPECIFICATIONS AND SAMPLING TESTING AND INSPECTION GUIDELINES.

- GRADING COMPLETE- All Shoulder Filling activities will be included and payed for as part of grading complete. This will include CONTRACTOR PROVIDING MATERIAL, material placement and all BMP’s(daily stabilization, fiber bonded matrix, permanent grassing, etc.). SHOULDER FILLING SHALL BE COMPLETED WITHIN 30 DAYS AFTER COMPLETION OF RESURFACING ROADWAY. ***MILLING OPERATION TO ADD 1FT. WIDENING ON RT SIDE WILL BE PAYED FOR AS PART OF GRADING COMPLETE.(SEE TYPICAL SECTION)

- 310-1101 GR AGGR BASE CRS- PLACE MATERIAL AT MAILBOXES BEFORE SHOULDER FILLING. APPROXIMATE AREA WILL BE 2’WIDTHX15’LENGTH AND 4”DEPTH OR AS DIRECTED BY ENGINEER

- CONTRACTOR SHALL MAINTAIN POSITIVE DRAINAGE THROUGHOUT PROJECT ENSURING ALL BMP’S ARE IN PLACE.

- PERFORM ALL PATCHING/WIDENING AS DIRECTED BY ENGINEER, PRIOR TO LEVELING AND RESURFACING OPERATIONS.

- ALL DRAINAGE STRUCTURES WITHIN THE LIMITS OF THE PROJECT WILL BE MAINTAINED THROUGHOUT THE DURATION OF THE PROJECT. CONTRACTOR WILL BE RESPONSIBLE FOR INSTALLING ALL BMP’S AT DRAINAGE STRUCTURES AND BE INCLUDED IN OVERALL PRICE.

- CONTRACTOR WILL BE RESPONSIBLE FOR FURNISHING AND INSTALLING

“LOW/SOFT SHOULDER” AND “ROAD WORK AHEAD”  SIGNS PER MUTCD AND SECTION 150, BEFORE ANY WORK BEGINS. PORTABLE SIGN CAN BE USED FOR DAILY OPERATION.

- THERMOPLASTIC WILL BE USED TO STRIPE ROADWAY.

- IN PREPARATION OF THE ROADWAY PRIOR TO RESURFACING, BLADE THE EXISTING SHOULDERS NO LOWER IN ELEVATION THAN THE EXISTING PAVEMENT OR SHOULDER. SHOULDER MATERIAL DISPLACED BY THE CLEANING OPERATION PRIOR TO RESURFACING AND/OR DURING MILLING OPERATION SHALL BE REMOVED DAILY AND DISPOSED OF AT AN APPROVED DISPOSAL SITE OFF TOWN/COUNTY RIGHT-OF-WAY OR AS DIRECTED BY ENGINEER. THE COST OF THIS WORK SHALL BE INCLUDED IN THE OVERALL BID PRICE.

- Contractors are expected to have appropriately sized equipment for the Contract and sufficient number of hauling trucks and manpower to keep asphalt spreading at a steady rate. The asphalt spreader must be equipped with skis to ensure a level drive surface. A roll pattern must be set up and proper types and sizes of rollers must be used in accordance with GDOT specs. Town OF HOMER reserves the right to shut down work immediately if equipment is inappropriately sized or insufficient for the job, or any aspect of the job is proving to be detrimental to the quality of the finished product.

- SEE 231 SPECIAL PROVISIONS BELOW FOR STREETS/DRIVEWAY TIE-ENDS.